Y Llawlyfr Arwyddion Traffig

Atodiad C i BENNOD 3 – Arwyddion Rheoleiddio

Amrywiadau Cymru

## Y Llawlyfr Arwyddion Traffig Pennod 3

# ATODIAD C: AMRYWIADAU CYMRU

## Cefndir

Mae'r Rheoliadau a Chyfarwyddydau Cyffredinol Arwyddion Traffig 2016 yn gymwys i Gymru, Lloegr a'r Alban. Yng Nghymru, cawsant eu diwygio ar 17 Medi 2023 gan Reoliadau a Chyfarwyddydau Cyffredinol Arwyddion Traffig (Diwygio) (Cymru) 2023 (2023 rhif 783). Gwnaed y newidiadau hyn o ganlyniad i benderfyniad Senedd Cymru i newid y terfyn cyflymder arferol ar ffyrdd ac arnynt oleuadau stryd i 20mya. Cafodd hyn ei wneud drwy newid ystyr Ffordd Gyfyngedig, fel y'i diffinnir yn adran 81(1) o Ddeddf Rheoleiddio Traffig Ffyrdd 1984, drwy Orchymyn Ffyrdd Cyfyngedig (Terfyn Cyflymder 20mya) (Cymru) 2022 (2022 rhif 800).

Mae'r newidiadau hyn yn effeithio ar y cyngor yn Llawlyfr Arwyddion Traffig Pennod 3 ynghylch arwyddion terfynau cyflymder. Eu heffaith ganlyniadol yw cyflwyno opsiynau ychwanegol ar gyfer arwyddion ysgolion a mesurau gostegu traffig, gan effeithio ar y Llawlyfr Arwyddion Traffig Pennod 4. Mae'r Atodiad hon yn ymdrin â'r ddwy bennod.

Mae'r rheolau sy'n llywodraethu gosod arwyddion terfyn cyflymder ategol mewn ardaloedd ac ynddynt oleuadau stryd wedi newid. Er na ellid gosod arwyddion fertigol neu farciau ffordd i ategu arwyddion 30mya yn flaenorol, bellach caniateir y rhain, gan fod eu hangen mewn amgylchiadau lle mae gorchymyn traffig wedi cael ei wneud i osod terfyn 30mya. I'r gwrthwyneb, ni chaniateir arwyddion 20mya ategol mewn ardal sydd â goleuadau stryd mwyach, gan mai'r goleuadau stryd eu hyn sy'n dangos y terfyn hwn.

## Newidiadau i Bennod 3 Adran 8 mewn perthynas â Chymru

O ganlyniad i'r newid i'r diffiniad o Ffordd Gyfyngedig, yn 8.1.2, disodli is-baragraff b) â:

Maximum speed limit, being an upper limit (see definition of “speed limit”). This includes restricted roads (sections 81 and 82 of the Road Traffic Regulation Act 1984), which are those roads with a speed limit of 20 mph by virtue of the presence of a system of street lighting where lamps are placed not more than 200 yards (183 metres) apart.

Yn 8.2.3, ac 8.3.1 i 8.3.3 disodli'r holl gyfeiriadau at ‘30mya’ ag ‘20mya’:

8.2.3  On a road with a 20 mph speed limit by virtue of street lighting (i.e. a restricted road), where the adjacent length of road is unlit and is subject to the national speed limit, the 20 mph speed limit will begin at the first lighting column. The terminal sign or signs should be placed on, or in line with, that column as appropriate. To ensure that drivers are aware of the commencement of the 20 mph limit, it is essential that the terminal signs are correctly placed and clearly visible. If the 20 mph limit is to start at some other point (before or after the street lighting begins), it will be necessary to make a speed limit order and locate the terminal signs accordingly. Where a 20 mph limit commences on an unlit section of road and continues as a restricted road on the lit section, additional terminal signs must not be placed, in either direction, at the point where the street lighting begins, i.e. where the 20 mph speed limit order ends and the restricted road begins (but see 8.3.2).

8.3.1  Whilst there is no specific requirement to provide repeater signs, it is for the traffic authority to determine how many are required and where they are placed. However, to ensure that drivers are fully aware of the speed limit in force it is recommended that repeater signs are provided at the intervals shown in Table 8-4 (see 8.3.4). Schedule 10 General Direction 2 prohibits the use of 20 mph repeater signs where a road has a system of carriageway lighting. Section 82 of the Road Traffic Regulation Act 1984 defines a lit road as being a “restricted road” (see 8.1.2). Section 81 specifies that the speed limit along such a road is 20 mph unless an order has been made to impose another limit and remove the restricted road status. The presence of carriageway lighting therefore means that a road automatically has a speed limit of 20 mph with the lamps taking the place of repeater signs. It is therefore important that, when lit roads have a speed limit other than 20 mph, repeater signs are provided at adequate intervals along the road as recommended in Table 8-4. Failure to do so could undermine confidence in the signing of all speed limits, as drivers have only the repeaters to tell them that the limit is not 20 mph. Where a speed limit, other than 20 mph, applies before and after the point where the carriageway lighting begins, a repeater sign should be placed in line with the first lamp; in most cases the sign would be mounted on the lighting column.

8.3.2  Where a 20 mph speed limit is imposed on an unlit road, it is necessary to make an order. In this case it is recommended that 20 mph repeater signs are provided (see 8.3.4). Where a road subject to a 20 mph limit is lit for only part of its length, it is recommended that a repeater sign for the unlit section when leaving the lit section should be placed at a distance of not more than 100 m from the last street lamp. Where two consecutive street lamps on a lit road are more than 183 m apart a speed limit order is required as it is not a “restricted road” with repeater signs provided where considered to be appropriate. Where the two lamps are less than 200 m apart it is unlikely that a repeater sign will be necessary.

8.3.3.  Schedule 10 General Direction 3 prohibits the use of repeater signs on an unlit road subject to the national speed limit. Where a lit road is subject to the national limit, repeater signs should normally be provided to remind drivers that the road is not a restricted road with a speed limit of 20 mph.

Yn Nhabl 8-4 mae'r pedair rhes gyntaf o wybodaeth yn cael eu disodli â:

| Speed limit and type of road1, 2 | Size of sign (mm) | Maximum distance (m) between |  | CVD (m) |
| --- | --- | --- | --- | --- |
|  |  | Each repeater sign | Terminal sign and first repeater3 |  |
| 20 mph lit road | - | No repeaters | No repeaters | - |
| 20 mph (unlit road) | 300 | 200 | 200 | 20 |
| 30 mph (lit or unlit road) | 300 | 250 | 200 | 30 |

## Parthau 20mya

Mae diwygiad Rheoliadau a Chyfarwyddiadau Cyffredinol Arwyddion Traffig 2023 yn diddymu arwyddion mewn parthau 20mya, gan fod goleuadau stryd yn yr ardaloedd hyn bron bob amser, ac felly bydd gan y ffyrdd hynny derfyn cyflymer yn rhinwedd eu statws ffordd cyfyngedig . Nid yw unrhyw arwyddion parth 20mya sy'n weddill bellach yn gyfreithlon a rhaid cael gwared arnynt.

Felly, nid yw adran 8.7 o Bennod 3 yn gymwys i Gymru.

## Rhybuddion dros dro am derfynau 20mya newydd

Ni chaniateir defnyddio'r arwydd dros dro ar gyfer terfyn 30mya newydd (diagram 7032, A13-6-35) yng Nghymru. Mae arwydd cyfatebol wedi'i ddarparu yn lle'r arwydd hwn, ond gyda geiriad mwy cryno a rhagor o hyblygrwydd o ran ei ddefnyddio. Nawr mae'r newid cenedlaethol i'r terfyn cyflymder ar ffyrdd ac arnynt oleuadau stryd yng Nghymru wedi digwydd, bydd yr angen am yr arwydd hwn yn gyfyngedig iawn. Mae ei ddefnyddio'n cael ei gyfyngu i sefyllfaoedd pan fo ffordd a oedd â therfyn cyflymder uwch o'r blaen newydd gael ei lleihau i 20mya – naill ai o ganlyniad i orchymyn traffig neu oherwydd bod goleuadau stryd wedi cael eu codi am y tro cyntaf.

Mae enw Adran 8.8 o Bennod 3 wedi cael ei newid i'r canlynol:

8.8 Warning of a new 20 mph speed limit or extending its length

Figure C-1 Diagram 7032.1 (S13‑6‑38A) (alternative versions)

8.8.1. The sign above may only be placed at the start of or in advance of any new 20 mph speed limit for a period of up to 12 months. The variant shown to the right is used in advance of the point where the restriction commences. The sign applies to a 20 mph limit when it first comes into force as a result of either of the following:

- roads that have become Restricted roads as a result of street lighting being installed on them and there being no other limit or order in force,

- roads that become subject to a 20 mph limit as a result of the making or revoking of a traffic order.

The normal regulatory signs must always be placed at the point where a 20 mph limit commences. Signs to diagram 7032.1 may be used in addition to remind the public of the country-wide change to the limit on street-lit roads, or of changes to locally set limits.

The version shown to the left is particularly useful at the point where an existing 20 mph limit has been extended into an area that previously had a higher limit. At such locations there will be no terminal sign, and Schedule 10 General Direction 2 (as amended) does not permit 20 mph repeater signs on lit roads. Regular drivers might not at first realise that the speed limit has changed and might not notice that the speed limit signs have been removed. A sign to diagram 7032.1 (S13‑6‑38A, shown above) should therefore be erected in these circumstances to advise drivers that a new 20 mph speed limit is in force. Figure 8-13 of Chapter 3 should be interpreted by reading speed limits signs showing “30” as “20”.

8.8.2.  Schedule 13 General Direction 13A sets out the conditions, all of which must be met, for the use of the sign to diagram 7032.1; they are:

a)  The sign must indicate only a new 20 mph speed limit (it must not be used to indicate any other speed limit);

b)  The sign must be removed not later than 12 months after the day on which the new speed limit comes into force.  It follows that any such signs installed in connection with the national change throughout Wales to the restricted road speed limit must have been removed before October 2024.

8.8.3.  The x‑height of the sign will depend on the location. In most cases the smallest size of 50 mm should be sufficient. A larger size might be required where the conspicuity of the sign needs to be increased, such as on a dual carriageway road. Only one sign, mounted on the nearside, will normally be needed although a second sign on the off side should be provided where a nearside sign is liable to be obscured (e.g. on a dual carriageway road). In accordance with S13‑7‑8, the sign must have, on its reverse, a date by which the person placing the sign reasonably believes the sign will have been removed.

## Arwyddion camerâu ac arwyddion terfyn cyflymder ategol

Mae paragraff 8.10.3 o Bennod 3 ynghyd â ffigur 8-17 yn cael eu disodli â’r canlynol:

Figure C-2 Diagram 880.2 (S11‑2‑64A) 
Speed camera ahead on a lit road and reminder of 20 mph speed limit

8.10.3.  On a lit road with a 20 mph speed limit, 20 mph repeater signs are not permitted. However, the sign to diagram 880.2 (above) may be used to advise drivers that a 20 mph speed limit is being enforced by cameras on roads that do not have repeater signs. This sign must not be used as a substitute for repeater or terminal signs, nor to indicate any other limit. The sign should normally be placed on the approach to an enforcement camera. For other speed limits or a 20 mph limit on an unlit road, a repeater sign to diagram 670 may be co‑located with the camera sign to diagram 878 without any legend as shown in Figure 8-19, especially on the approach to an enforcement camera.

## Marciau terfyn cyflymder ar y ffyrdd

Mae paragraff 8.18.1 o Bennod 3 yn cael ei ddisodli â’r canlynol:

8.18.1  The road marking roundels shown in diagram 1065 (S10‑2‑9, see Figure 8-22) may be used to supplement upright signs, or on their own as repeater signs. The road marking roundel must not be used on its own where a speed limit commences; if provided it must be used in conjunction with an upright terminal sign to diagram 670 or 676 (Schedule 10 General Direction 5). As with the upright sign, the marking must not be used as a repeater on a lit road subject to a 20 mph speed limit (Schedule 10 General Direction 2, as amended). The larger marking should be used at the start of a speed limit if approaching traffic is subject to a limit higher than 40 mph, otherwise the smaller size should be used. The size of repeater markings should be appropriate to the speed indicated, with the smaller roundel being used where the limit is 40 mph or less.

## Darpariaethau Arbed

Gallai arwyddion sydd eisoes wedi'u gosod yn gyfreithiol pan ddaeth diwygiad 2023 i rym aros tan 17 Medi 2024, pe bai'r wybodaeth a ddangosir arnynt yn dal i fod yn gywir, ond erbyn hyn rhaid bod wedi cael gwared arnynt. Mae un eithriad i hyn:

the speed limit road marking (diagram 1065, S10-2-9) showing the digits “20” in a street-lit area may remain until September 2028,

Gellir defnyddio'r marc ffordd yn niagram 1065 sy'n dangos terfyn ar wahân i 20mya ar unrhyw ffordd yng Nghymru. Yn y sefyllfa anarferol pan fo ffordd heb oleuadau sydd â therfyn cyflymder o 20mya yn rhinwedd gorchymyn traffig, byddai hefyd yn parhau i fod yn briodol defnyddio'r marcio gyda'r digidau ‘20’.

Nid yw'r ddarpariaeth arbed hon yn caniatáu i arwydd neu farc ffordd barhau sy'n dangos y terfyn cyflymder anghywir ar gyfer y ffordd dan sylw.

## Opsiynau ychwanegol ar gyfer arwyddion rhybuddio ac arwyddion gwybodaeth

## Amrywiadau sy'n ymwneud â'r Llawlyfr Arwyddion Traffig Pennod 4

Mae diddymu arwyddion parth 20mya yng Nghymru yn cael dwy effaith ganlyniadol:

- Mae'n bosibl y bydd angen arwyddion ychwanegol ar gyfer mesurau tawelu traffig ffisegol sy'n bresennol yn y parthau hyn i rybuddio am eu presenoldeb. Mae hyn er mwyn cydymffurfio â Rheoliadau Priffyrdd (Twmpathau Ffyrdd) 1999 (1999 rhif 1025) (HRHR), nad ydynt wedi'u diwygio.

- Mae'r opsiwn i gynnwys panel is ar arwyddion wrth gyrraedd y parth yn dangos gwaith celf plant neu waith celf arall wedi cael ei golli.

Er mwyn lliniaru'r effeithiau hyn, cyflwynwyd arwyddion newydd lle mae modd ychwanegu plât ar y gwaelod sy'n cynnwys graffig o ddraig. Mae hyn yn seiliedig ar y cynllun a enillodd gystadleuaeth i blant a gafodd ei threfnu gan Lywodraeth Cymru a RoSPA. Mae'r arwyddion hyn yn ychwanegol at y rhai sydd eisoes wedi'u rhagnodi, ac maent ar gael fel opsiwn i awdurdodau eu defnyddio yn ôl eu disgresiwn.

## Rhybuddio am dwmpathau ffordd

Mae Rheoliad 6 o Reoliadau Priffyrdd (Twmpathau Ffyrdd) yn ei gwneud yn ofynnol i arwyddion traffig gael eu gosod a'u cynnal mewn safleoedd y mae'r awdurdod yn ystyried eu bod yn ofynnol darparu rhybudd digonol o bresenoldeb twmpath(au) ffordd. Nid yw'r gofyniad hwn yn berthnasol o fewn parthau 20mya. Nid yw Rheoliadau Priffyrdd (Twmpathau Ffyrdd) yn sôn am derfynau cyflymder 20mya nad ydynt yn barthau, gan fod y rhain yn brin iawn pan gawsant eu gwneud. Felly, gall awdurdodau traffig ystyried bod ffordd sy'n ddarostyngedig i derfyn cyflymder 20mya yn cyfateb i barth 20mya, ac nad oes angen unrhyw arwyddion o gwbl i rybuddio am dwmpathau ar ffordd o'r fath.

Fodd bynnag, os yw'r awdurdod traffig yn dod i'r casgliad bod angen arwyddion rhybuddio, cânt ddefnyddio'r opsiynau a oedd ar gael yn Rheoliadau a Chyfarwyddiadau Cyffredinol Arwyddion Traffig 2016, neu ddefnyddio arwyddion wedi'u rhagnodi yn niwygiad Rheoliadau a Chyfarwyddiadau Cyffredinol Arwyddion Traffig 2023.

Disgrifir y defnydd o arwyddion rhybuddio fel a ragnodir yn Rheoliadau 2016 yn Adran 12 o Lawlyfr Arwyddion Traffig Pennod 4, gyda'r amod y dylid darllen cyfeiriadau at ‘Diagram 557.1 (S2-2-43)’ fel ‘Diagram 557.2 (S2-2-43A)’.

| Figure C-3 Diagram 557.2 (S2‑2‑43A) showing plate variant (e) | Figure C-4 Diagram 883.1 (S11‑2‑32A) |
| --- | --- |

Mae ffigurau C-3 a C-4 yn dangos arwyddion ac opsiynau a ragnodwyd gyntaf gan ddiwygiad Rheoliadau a Chyfarwyddiadau Cyffredinol Arwyddion Traffig 2023. Bwriad y rhain yw darparu rhybudd ar draws ardal gyfan a allai gynnwys rhwydwaith o ffyrdd cysylltiedig, yn debyg i un yr hen arwyddion wrth gyrraedd a gadael parth 20mya. Bydd hyn yn osgoi'r angen i osod arwyddion rhybuddio ar bob ffordd neu cyn pob grŵp o dwmpathau. Bydd yr arwyddion hyn fel arfer yn cael eu gosod ar y pyst lle roedd yr arwyddion parth, gan arbed cost ac osgoi amharu. Rhaid iddynt adlewyrchu golau ond nid oes angen eu goleuo.

Yn anaml iawn y bydd angen yr arwydd yn Ffigur C-4 (Diagram 883.1, S11‑2‑32A). Fel arfer bydd yn amlwg i ddefnyddwyr y ffordd pan fydd y mesurau gostegu traffig wedi dod i ben. Hyd yn oed os nad yw'n amlwg, ni fyddai'n fater diogelwch pe baent yn parhau i yrru'n ofalus. Pan ddefnyddir yr arwydd hwn, bydd y maint llai (40mm x-uchder) fel arfer yn ddigonol.

## Arwyddion rhybuddio a negeseuon diogelwch ysgolion

Mae rheoliadau diwygio 2023 yn rhoi'r opsiwn ar gyfer plât sy'n dangos delwedd graffig i'w osod o dan naill ai'r arwydd rhybuddio am blant neu'r arwydd rhybuddio am dwmpathau. Yn unol â hynny, cyfeirir at y ddau arwydd rhybuddio hyn fel yn Diagram 545.2 (S2-2-25A) a Diagram 557.2 (S2-2-43A), yn y drefn honno. Maent yn darparu ar gyfer y plât ychwanegol fel opsiwn drwy'r cyfeirnod ‘5’ yng ngholofn 5 y tabl. Dim ond yn ychwanegol at y plât gofynnol arall y gellir defnyddio'r plât hwn; ni ddylid ei osod yn lle'r plât arall.

Mae'r plât dewisol hwn, sy'n cynnwys delwedd o ddraig wedi'i siapio i fod yn debyg i'r digidau ‘20’ yn seiliedig ar waith celf plentyn, yn Niagram 545.3 (S2-3-5). Rhagnodir un maint ar ei gyfer: 545mm o led gan 410mm o uchder. Mae hyn yn cyfateb i led y plât â geiriau uwch ei ben ar ei x-uchder mwyaf cyffredin: 62.5mm. Pwrpas y plât yw gwella diogelwch ar y ffyrdd. Mae'r platiau wedi'u gwahanu'n fertigol gan oddeutu 30mm ar gyfer y triongl arferol 600mm o uchder.

Dangosir trefniadau nodweddiadol ar gyfer defnyddio'r opsiwn hwn yn Ffigur C-5. Gellir gosod y cyfan ar fwrdd cefndir melyn lle ystyrir bod angen eglurder ychwanegol.

Figure C-5 School and humps warning signs
with additional plate to Diagram 545.3 (S2‑3‑5)